# Introduction Exercise

# Section 2

## Activity

- Modify the L-Bracket provided to a width of 100mm. Then register on the 3DContentCentral website and download any part.

## Instruction

- Open L-Bracket.

- Modify the width.

  - Double click the part in the graphics area.

  - Double click the Width Dimension label on the part.

  - Change the width to 100mm in the Modify window.

  - Click OK.

- 3DContentCentral

  - Open the 3DContentCentral website.

  - Click Register in the top left corner.

  - Follow the registration instructions.

  - After registration, login to 3DContentCentral, located in the top left corner.

  - Select a part and download it.

# Section 3

## Activity

Create a Customized Unit System and a Customized Drafting Standard.

## Instruction

- Open L-Bracket.

- Create a Customized Unit System.

  - Open the Tools dropdown menu.

  - Select Options.

  - Open the Document Properties tab.

  - Click Units.

  - Under Unit System, select Custom.

  - Select the desired units for each measurement type them in the table.

- Select an Overall Drafting Standard.

  - Open the Tools dropdown menu.

  - Select Options.

  - Under the Document Properties tab, click Drafting Standard.

  - Select the desired overall drafting standard.

- Customize a Drafting Standard

  - Open the Tools dropdown menu.

  - Select Options.

  - Open the Document Properties tab.

  - Explore any category along the left and modify their properties.