# Lesson 2 Exercises

# Section 1

## Activity

Investigate all of the mating techniques provided in SolidWorks.

## Instruction

- Open the assembly used in the Lesson 2 Section 1 video.

- Create a table and list every mate provided by SolidWorks.

- Describe each mate. For more information use SolidWorks Help.

| Mate | Description |
| --- | --- |
| Coincident | Make the two selected surfaces co-planar |
| … |  |
| … |  |

# Section 2

## Activity

Create the following Kicker Assembly:

## Instruction

- Insert the Bearing Mount, two Bearing Housings, and a Bearing.

- Mate two Bearing Housings together and mate the Bearing through their center.

- Insert the Main Shaft and mate it with the Bearing.

- Insert two Shaft Brackets and two Kicking Shafts.

- Mate the Shaft Brackets and the Kicking Shafts together. Then mate them with the Main Shaft.

- Insert the Gear Bracket, Gear Driven, and Gear Assembly Weld.

- Mate the Gear Bracket, Gear Driven and Gear Assembly Weld together. Then mate them to the Main Shaft.

- Repeat steps 1-3 for the other side and mate it with the rest of the assembly.

# Section 3

## Activity

Improve the assembly structure of the Kicker Assembly.

## Instruction

- Regroup the parts into the following sub-assemblies. Create the sub-assemblies only.

  - Bearing Support Sub-assembly

    - Bearing Mount

    - Bearing Housing x 2

    - Bearing

  - Gear Sub-assembly

    - Gear Driven

    - Gear Assembly Weld

    - Gear Bracket

  - Kicking Shafts Sub-assembly

    - Main Shaft

    - Shaft Bracket x 2

    - Kicker Shaft x 2

# Section 4

## Activity

Create an exploded view of the Kicker Assembly and then add exploded lines.

## Instruction

- Create an exploded view by using the Exploded View feature.

- Create an exploded view line by using Explode Line Sketch.

# Section 5

## Activity

Use the sub-assemblies created in the Section 3 exercise to create a new Kicker Assembly with a variety of sub-assemblies rather than individual parts.

## Instruction

- Insert the Gear Sub-assembly, Bearing Support Sub-assembly, Kicking Shafts Sub-assembly, and two Bearing Support Sub-assemblies.

- Mate them together to create the Kicker Assembly.

# Section 6

## Activity

Create a fly-by animation of the Kicker Assembly while it is rotating.

## Instruction

- Rotational animation

  - Open Motion Study tab.

  - Rotate the kicker by using the Motor tool.

- Camera-based animation

  - Create a path for the camera using a sketch.

  - Enable Camera View.

  - Add a camera and configure its properties.

- Produce as an AVI.

  - Produce the animation as an AVI by clicking the Save Animation button.