# Lesson 1 Exercises

# Section 1

## Activity

Review all the different types of Orientation views and review how to move the part in the graphics area. Then create a cross-sectional view.

## Instruction

- Select the Orientation, Navigation and Sections view.

  - Open the L-Bracket part provided.

  - Go through each orientation view.

- Move the part.

  - Rotate

  - Pan

  - Roll View

  - Zoom

- Cross-sectional view

  - Click Section View.

  - Select the plane from the Section View Property Manager.

  - Click and drag the arrow to the desired depth.

  - Click OK.

  - Click Section View once again to exit the Cross-sectional view.

# Section 2

## Activity

Describe each feature of L-Bracket part provided.

## Instruction

- Click a feature under Feature Manager Design Tree to highlight this feature on the model.

# Section 3

## Activity

Create an L-Bracket part. The details and dimensions are provided in the following drawing:

## Instruction

- Create the L-shaped sketch on the right plane.

- Extrude the sketch into a 3D part.

- Add the Fillet feature by using Fillet Tool.

- Create two small holes by using the Hole Wizard.

  - Click Hole Wizard under the Features tab on the Command Manager.

  - Under Hole Type, click the Hole icon.

  - Under Hole Type, select ISO standard.

  - Under Hole Type, select Drill Size as the type.

  - Under Hole Specifications, select ∅6.0.

  - Under End Condition, select Through All.

  - Click the Positions tab on top of the property manager.

  - Place two holes on the bottom plate.

  - Constrain them with the Smart Dimension tool.

  - Click OK.

- Create one large hole on the L-bracket using the Extruded Cut tool.

- Remove the corners of the L-bracket by using Extruded Cut.