# Lesson 1 Section 3, Exercise Video Solution

Welcome to SolidWorks for the engineering design process, Lesson 1 Section 3 exercise solution.

This video will guide you through the steps needed to create this L-Bracket part with the dimensions shown on this drawing.

Click new, select part, and click OK.

# Creating the Sketch

Click Extruded Boss/Base, and select the right plane. The sketch toolbar appears in the command manager, since a sketch is required before an extrusion. Select the line tool and create a horizontal line starting at the origin. Now create a vertical line starting at the end of the first line.

Select Smart dimension from the command manager, select the vertical line, [pause] and set the dimension to 50.8 mm. Click the horizontal line and set its length to 25.4 mm. Click the green check mark to close the property manager.

Now, you will use the Offset Entities tool to create the inner edges of the L bracket. Click Offset Entities and then click one of the lines. Click reverse to correct the direction of the offset. Specify the thickness of the L bracket to be 3.17 mm and click the green check mark. To complete the sketch, use the Line tool to connect the open sections of the L bracket and click the green check mark once you are done.

# Creating the Extrusion

Click exit sketch and the extrude property manager appears. Set the end condition to mid-plane and set the depth to be 50.8 mm. Click the green check mark once you are done.

# Fillets

You will now create a rounded edge for this inner edge of the L-bracket. Click Filet and then click this inner edge. Set the radius to 6.35 mm and click the green check mark.

# Holes

You will now add two holes to this L-bracket. Click Hole wizard in the command manager. Under Hole type, click the hole icon, select the ISO standard, and set the type to be “drill sizes”. Under Hole specifications, set the size to 6 mm diameter. Under end condition, select through all.

Now click the position tab near the top of the property manager. Click to place two holes on the bottom plate.

Click smart dimension on the command manager and constrain the holes to the appropriate dimensions. Click the green check mark once you are done.

You will now add the hole that goes on the rear plate of the L-bracket. This will be done using the extruded cut feature rather than the hole wizard because the diameter is 12.70 mm. This is not a standard hole size, so the hole wizard cannot be used. Click Extruded cut on the command manager and select the surface. Select the circle tool from the sketch toolbar and draw the circle as show. Use the smart dimension tool the set the diameter to 12.70 mm. Take this opportunity to move this circle to the correct location. Use the smart dimension tool to set the distance between the center of this circle and the bottom of the L-bracket to 38.10 mm.

Click Exit sketch and the Cut-Extrude property manager appears. Set the end condition to through all and click the green check mark to apply the cut.

# Shaping the L-Bracket

You will now create a rounded V-shape onto this L-bracket to match what is shown on the drawing. Click Extruded cut on the command manager and select this surface.

Switch to the front view to make it easier to create the sketches. Select the circle tool, click the center of the hole you just created, and then click the top edge. This will create the 25.4 mm diameter circle specified in the drawing.

Now select the line tool. Create a line from this bottom left corner to the outer edge of the circle. Do this for the other side as well.

Click Exit sketch and the cut-extrude property manager appears. For the end condition, select through all. For the selected contours, select the two corners of the L-bracket and click the green check mark to apply the cut.

# End

In this video, you created the L-bracket part from the exercise.